# Database Consistency Test

# Introduction

This EngineeringPaper.xyz sheet shows how to calculated the maximum stress and displacement for a cantilever beam loaded by a vertical force at its end. The geometry of the beam is shown in the figure below. The beam has a rectangular cross section with height h and width b. The maximum stress in a cantilever with length much larger than its height is due to bending stress that occurs at the base of the beam. For a downward force, the maximum stress will be a tensile force at the top of the beam and a compressive stress at the bottom of the beam. For beam sections that are symmetric top to bottom, the maximum tensile and compressive stresses will have equal magnitude. #  # # Select Beam Material

# Select Beam Section

Source for beam section equations and images: https://en.wikipedia.org/wiki/List_of_second_moments_of_area

# Define the Other Beam Parameters

l=500mm

F=500N

# Define the Deflection and Stress Equations

The maximum displacement of a cantilever beam loaded at its end is defined by [1]:

ymax=−F⋅l33⋅E⋅Ix

where Ix is the area moment of inertia of the cross section about the x-axis as selected in the table above. The maximum displacement can now be queried:

ymax=mm=−0.165297794307557mm

The maximum bending stress in a beam section is given by:

σmax=M⋅cIx

where M is the bending moment at the section of interest and c is distance from the neutral axis of the beam to the top or bottom of the beam. The maximum moment occurs at the base of the beam and is defined by:

M=l⋅F

where l is the length of the beam. Now that everything is defined, the maximum stress can be queried and converted to [MPa] units:

σmax=MPa=5.33333333333333MPa

These results can be compared to the results obtained from a finite element analysis using SolidWorks. The beam has the same dimensions, loading, and material as the rectangular cross-section and the aluminum table options above. The displacement plot is shown below. The maximum displacement magnitude obtained using finite element analysis is .175 [mm], which is close to the .165 [mm] obtained using the equations above.

Similarly, the maximum stress calculation can be compared to the SolidWorks results. The maximum stress obtained from SolidWorks is 6.5 [MPa] as shown below. Note that this is higher than the 5.3 [MPa] obtained using the beam equation. This is the result of the stress concentration that occurs at the based of the cantilever where it meets the fixed boundary condition. A littler further from the fixed boundary condition, the finite element values are close to the calculated values.

[1] Norton, R. L. “Machine design. A integrated approach, 5th Editi.” (2013).

y=F6⋅E⋅Ix⋅x3−3⋅l⋅x2

# Test 16Created with EngineeringPaper.xyz